Safety Data Sheet

Preparation Date 05-May-2015	Revision date 09-Sep-2020	Revision Number: 1

1. Identification of the Substance/Preparation and of the Company/Undertaking

## Identification of the product

Product Description	Pendi Hydrocap

## Other means of identification

Product code	12-230AA

UN/ID no	UN3082

Registration number(s)	70506-230-72159

Recommended use of the chemical and restrictions on use Recommended use	Pre-emergence weed control.

Uses advised against	Activities contrary to label recommendation

## Details of the Supplier of the Safety Data Sheet Supplier Address

Agrisel USA, Inc

PO Box 3528

Suwanee, GA 30024

## Emergency telephone number

Company Phone Number	1-770-831-4160

Emergency telephone number	Chemtrec: (800) 424-9300 (24hrs) or (703) 527-3887Medical: Rocky Mountain Poison and

2. Hazards Identification

Drug Safety	(866) 673-6671 (24hrs)

## Classification

OSHA Regulatory Status

This chemical is considered hazardous by the 2012 OSHA Hazard Communication Standard (29 CFR 1910.1200)

| Acute toxicity - Inhalation (Vapors) | Category 3 |
| --- | --- |
| Skin sensitization | Category 1 |

## Label elements

EMERGENCY OVERVIEW

DANGER

Hazard Statements

Toxic if inhaled

May cause an allergic skin reaction

Appearance yellow-orange

Physical state Liquid

Odor faint

Precautionary Statements - Prevention

Do not get in eyes, on skin, or on clothing Use only outdoors or in a well-ventilated area

Contaminated work clothing should not be allowed out of the workplace Wear protective gloves

IF ON SKIN: Wash with plenty of soap and water

If skin irritation or rash occurs: Get medical advice/attention Wash contaminated clothing before reuse

IF INHALED: Remove victim to fresh air and keep at rest in a position comfortable for breathing Call a POISON CENTER or doctor/physician

## Precautionary Statements - Storage

Store in a well-ventilated place. Keep container tightly closed Store locked up

## Precautionary Statements - Disposal

Dispose of contents/container to an approved waste disposal plant

## Hazards Not Otherwise Classified (HNOC) OTHER INFORMATION

- Very toxic to aquatic life with long lasting effects

- Very toxic to aquatic life

- May be harmful in contact with skin

3. Composition/information on Ingredients

| Chemical name | CAS No | Weight-% |
| --- | --- | --- |
| Pendimethalin [N-(1-ethylpropyl)-3,4-dimethyl-2,6-dini trobenzeneamine] | 40487-42-1 | 38.7 |

If CAS number is "proprietary", the specific chemical identity and percentage of composition has been withheld as a trade secret.

4. First aid measures

## FIRST AID MEASURES

Eye contact	Hold eye open and rinse slowly and gently with water for 15 - 20 minutes. Remove contact lenses, if present, after 5 minutes, then continue rinsing eye. Call a poison control center or doctor for treatment advice.

Skin contact	Rinse skin immediately with plenty of water for 15-20 minutes. Call a poison control center or doctor for treatment advice. Remove and wash contaminated clothing before re-use.

Inhalation	Move person to fresh air. If person is not breathing, call 911 or an ambulance, then give artificial respiration, preferably mouth-to-mouth if possible. Call a physician or poison control center immediately.

Ingestion	Call a physician or poison control center immediately. Never give anything by mouth to an unconscious person. Do not induce vomiting without medical advice.

## Most Important Symptoms and Effects, Both Acute and Delayed

Most Important Symptoms and Effects

no data available.

## Indication of Any Immediate Medical Attention and Special Treatment Needed

Notes to physician	Treat symptomatically.

5. Fire-fighting measures

## Suitable extinguishing media

Use extinguishing measures that are appropriate to local circumstances and the surrounding environment.

Unsuitable extinguishing media no data available.

## Specific hazards arising from the chemical

No information available.

Hazardous combustion productsCarbon monoxide. Carbon dioxide (CO2). Oxides of nitrogen. Hydrocarbons.

## Explosion data

Protective equipment and precautions for firefighters

Use personal protective equipment. As in any fire, wear self-contained breathing apparatus pressure-demand, MSHA/NIOSH (approved or equivalent) and full protective gear.

6. Accidental release measures

## Personal precautions, protective equipment and emergency procedures

Personal Precautions	Avoid contact with skin and eyes. Wear protective gloves/protective clothing and eye/face

protection. Wash thoroughly after handling.

## Environmental Precautions

Environmental precautions	Consult a regulatory specialist to determine appropriate state or local reporting

requirements, for assistance in waste characterization and/or hazardous waste disposal and other requirements listed in pertinent environmental permits.

## Methods and material for containment and cleaning up

Methods for Clean-Up	Soak up with inert absorbent material (e.g. sand, silica gel, acid binder, universal binder,

sawdust). Sweep up and shovel into suitable containers for disposal.

7. Handling and Storage

## Precautions for safe handling

Handling	Pesticide applicator and workers must follow the label and Direction for Use in accordance with the EPA Worker's Protection Standard. For manufacturing, blending or packaging the following recommendations are provided:. Avoid contact with skin and eyes. Keep away from open flames, hot surfaces and sources of ignition. Do not eat, drink or smoke when using this product. Provide adequate ventilation. Remove all sources of ignition. Wash thoroughly after handling. Keep out of reach of children.

## Conditions for safe storage, including any incompatibilities

Storage	Keep containers tightly closed in a cool, well-ventilated place.

incompatible materials	No information available.

8. Exposure Controls/Personal Protection

Exposure guidelines	This product does not contain any hazardous materials with occupational exposure limits

established by the region specific regulatory bodies.

Engineering controls	Investigate engineering techniques to reduce exposures. Local mechanical exhaust

ventilation is preferred. Consult ACGIH ventilation manual or NFPA Standard 91 for design

of exhaust systems.

## Personal protective equipment

Eye/Face Protection	Where there is potential for eye contact have eye flushing equipment available. Use eye protection to avoid eye contact.

Skin protection	Long sleeved clothing. Chemical resistant gloves.

Respiratory protection	Where airborne exposure is likely, use NIOSH approved respiratory protection equipment appropriate to the material and/or its components. Full facepiece equipment is recommended and, if used, replaces need for face shield and/or chemical goggles. If exposures cannot be kept at a minimum with engineering controls, consult respirator manufacturer to determine appropriate type equipment for given application. Observe respirator use limitations specified by NIOSH or the manufacturer. For emergency and other conditions where there may be a potential for significant exposure, use an approved full face positive-pressure, self-contained breathing apparatus. Respiratory protection programs must comply with 29 CFR 1910.134.

## General hygiene considerations

Do not eat, drink or smoke when using this product. Remove and wash contaminated clothing before re-use. Avoid contact with skin, eyes and clothing. Wear suitable gloves and eye/face protection. Wash hands and face before breaks and immediately after handling the product.

9. Physical and Chemical Properties

## Information on basic physical and chemical properties

| Physical state Appearance color Property | Liquid yellow-orange No information available VALUES | Odor faint Remarks/ • Method |
| --- | --- | --- |
| pH Melting point/freezing point Boiling Point/Range Flash Point Evaporation Rate Flammability (solid, gas) | 7.72 8.72 °C / 48 °F No information available No information available No information available No information available |  |
| Flammability limit in air |  |  |
| Upper Flammability Limit Lower Flammability Limit vapor pressure Vapor Density Specific gravity Water solubility Solubility in Other Solvents | No information available No information available No information available No information available 1.1705@ 20 C No information available No information available |  |

Partition coefficient: n-octanol/waterNo information available Autoignition temperature	no data available Decomposition temperature	No information available Viscosity, kinematic	No information available

Dynamic viscosity	No information available

Explosive properties	No information available

Oxidizing properties	No information available

## OTHER INFORMATION

Softening point	No information available

molecular weight	No information available

VOC Content	No information available

Liquid Density	No information available

10. Stability and Reactivity

## Reactivity

no data available

## Chemical stability

Stable under recommended storage conditions.

## Possibility of hazardous reactions

None under normal processing.

## Conditions to avoid

Extremes of temperature and direct sunlight.

## incompatible materials

No information available.

## Hazardous decomposition products

None under normal use conditions.

11. Toxicological Information

## Information on Likely Routes of Exposure

Product information	Pendimethalin 38.7% = Oral LD50 (rat) = >5,000 mg/kgDermal LD50 (rat/rabbit) = >5,000

mg/kg Inhalation LC50 (rat) = >5.39 mg/L (Max achievable breathing zone) 4 hrPrimary eye irritation (rabbit) = Not an eye irritantPrimary dermal irritation (rabbit) = Not a skin irritantDermal Sensitization = Not a sensitzer

Inhalation	May cause irritation of respiratory tract.

Eye contact	Moderately irritating to the eyes.

Skin contact	May be harmful if absorbed through the skin.

Ingestion	MAY BE HARMFUL IF SWALLOWED.

## Information on Toxicological Effects

Symptoms	No information available.

## Delayed and immediate effects as well as chronic effects from short and long-term exposure Sensitization	No information available.

Mutagenic effects	no data available.

Carcinogenicity	The information below indicates whether any agency has listed any ingredient as a

carcinogen.

Reproductive effects	Not Available.

STOT - Single Exposure	no data available.

STOT - Repeated Exposure	no data available.

Aspiration hazard	No information available.

Numerical Measures of Toxicity - Product information LD50 Oral	>  5000  mg/kg (rat)

LD50 Dermal	>  5000  mg/kg (rat)

LC50 Inhalation	Inhalation LC50 > 5.39 mg/l

12. Ecological Information

IMDG - Marine Pollutant.

## ecotoxicity

Pendimethalin:

Rainbow trout 96 hr LC50 = 0.89 mg/L Daphnia Magna 48 hr EC50 = 0.979 mg/L Green algae 72 hr EC50 = 0.0081 mg/L Mallard duck LD50 = 1421 mg/kg

Honey Bee LD50 = 49.8 ug/bee

## Persistence/Degradability

no data available.

## Bioaccumulation/ Accumulation

No information available.

## Other Adverse Effects

no data available

13. Disposal Considerations

## Waste Treatment Methods

Waste Disposal Method	Pesticide wastes are acutely hazardous. Improper disposal of excess pesticide or rinsate is

a violation of Federal law. If the wastes cannot be disposed of by use or according to label instructions, contact your State Pesticide or Environmental Control Agency, or the Hazardous Waste representative at the nearest EPA Regional Office for guidance.

Contaminated packaging	Refer to product label.

14. Transport Information

DOT	Not regulated when shipped domestically by highway in non-buk containers as per 49 CFR 171.4

UN/ID no	UN3082

Proper shipping name	Environmentally hazardous substance, liquid, n.o.s (Pendimethalin)

## Hazard class	9

Packing group	PG III

IMDG - Marine Pollutant	IMDG - Marine Pollutant.

TDG	When shipped in Canada domestic highway non-bulk this product can be shipped as Not regulated as per TDG 1.45.1

UN/ID no	UN3082

Proper shipping name	Environmentally hazardous substance, liquid, n.o.s. (Pendimethalin)

## Hazard class	9

Packing group	PG III

## Marine Pollutant: Y or N [List Ingredient(s) with %]

Marine pollutant.

## ICAO

UN/ID no	UN3082

Proper shipping name	Environmentally hazardous substance, liquid, n.o.s (Pendimethalin)

## Hazard class	9

Packing group	PG III

Description	IMDG - Marine Pollutant

## IATA

UN/ID no	UN3082

Proper shipping name	Environmentally hazardous substances, liquid, n.o.s (Pendimethalin)

## Hazard class	9

Packing group	PG III

Description	IMDG - Marine Pollutant

## IMDG

UN/ID no	UN3082

Proper shipping name	Environmentally hazardous substances, liquid, n.o.s (Pendimethalin)

## Hazard class	9

Packing group	PG III

Environmental hazards	IMDG - Marine Pollutant

15. Regulatory Information

This chemical is a pesticide product registered by the Environmental Protection Agency and is subject to certain labeling requirements under federal pesticide law. These requirements differ from the classification criteria and hazard information required for safety data sheets, and for workplace labels of non-pesticide chemicals. Following is the hazard information as required on the pesticide label:

signal word	CAUTION

Ventilation Control	PESTICIDE APPLICATORS & WORKERS THESE WORKERS MUST REFER TO

PRODUCT LABELING AND DIRECTIONS FOR USE IN ACCORDANCE WITH EPA WORKER PROTECTION STANDARD 40 CFR PART 170.

## Causes moderate eye irritation. Harmful if swallowed or absorbed through the skin. Toxic to fish.

International Inventories

USINV	Not present

DSL/NDSL	Not present

## EINECS/ ELINCS

Not Present

ENCS	Not Present

China	Not Present

KECL	Not Present

PICCS	Not Present

AICS	Not Present

TSCA	Not Present

TSCA - United States Toxic Substances Control Act Section 8(b) Inventory

DSL/NDSL - Canadian Domestic Substances List/Non-Domestic Substances List

EINECS/ELINCS - European Inventory of Existing Commercial Chemical Substances/EU List of Notified Chemical Substances

ENCS - Japan Existing and New Chemical Substances IECSC - China Inventory of Existing Chemical Substances KECL - Korean Existing and Evaluated Chemical Substances

PICCS - Philippines Inventory of Chemicals and Chemical Substances

AICS - Australian Inventory of Chemical Substances

# Federal Regulations

## SARA 313

Section 313 of Title III of the Superfund Amendments and Reauthorization Act of 1986 (SARA). This product contains a chemical or chemicals which are subject to the reporting requirements of the Act and Title 40 of the Code of Federal Regulations, Part 372:

## CERCLA

Not applicable

## CERCLA

SARA Product RQ	0

RCRA

Pesticide Information

State Regulations

State Right-to-Know

| Chemical name | Massachusetts | New Jersey | Pennsylvania | Illinois | Rhode Island |
| --- | --- | --- | --- | --- | --- |
| Pendimethalin [N-(1-ethylpropyl)-3,4-dimethyl -2,6-dinitrobenzeneamine] - 40487-42-1 |  | X |  |  |  |

International regulations

U.S. EPA Label information

EPA Pesticide registration number 70506-230

16. Other Information

## NFPA	HEALTH 1	flammability 1	Instability 0	Physical hazard -

Preparation Date	05-May-2015

Revision date	05-Sep-2019

Revision Summary Update logo Disclaimer

## Agrisel USA, Inc. believes that the information and recommendations container herein (including data and statements) are accurate as of the date hereof. NO WARRANTY OF FITNESS FOR ANY PARTICULAR PURPOSE, WARRANTY OF MERCHANTABILITY, OR ANY OTHER WARRANTY, EXPRESSED OR IMPLIED, IS MADE CONCERNING THE

INFORMATION PROVIDED HEREIN. The information provided herein relates only to the specific product designated and may not be valid where such product is used in combination with other materials or in any process. Further, since the conditions and methods of use are beyond the control of Agrisel USA, Inc. and Agrisel USA, Inc. expressly disclaims any and all liability as to any results obtained or arising from any use of the product or reliance on such information.

End of SDS